Nikon Digital Camera COOLPIX P1/P2 Specifications

Type: 	Compact digital camera

Effective pixels: 	8.0 million (P1)/ 5.1 million (P2)

CCD: 	P1: 1/1.8-inch type (8.31 million total pixels) / P2: 1/1.8-inch type (5.26 million total pixels)

Image modes: 	P1: 8M (3264), 5M (2592), 3M (2048), 2M (1600), 1M (1280), PC (1024), TV (640), 3:2 (3264) / P2: 5M (2592), 3M (2048), 2M (1600), 1M (1280), PC (1024), TV (640), 3:2 (2592)

Lens: 	3.5x Zoom-Nikkor; 7.5-26.3mm (35mm [135] format equivalent to 36-126mm); f/2.7-5.2; 7 elements in 6 groups; Digital zoom: up to 4x

Focus range: 	50cm (19.6 in.) to infinity (∞), 4cm (1.6 in.) to infinity (∞) in Macro Mode

LCD monitor: 	2.5-inch type, 110,000-dot TFT LCD monitor with brightness adjustment

Storage media: 	Internal memory (P1: approx. 32MB, P2: approx 16MB), SD memory card

Shooting modes: 	Auto, Programmed Auto, Aperture-priority Auto Mode, Scene (Panorama assist, Beach/Snow, Museum, Copy, Sports), Advanced Scene (Portrait [Face AF], Portrait, Party/Indoor, Night portrait, Landscape, Night landscape, Sunset, Dusk/Dawn, Fireworks show, Back light, Close up), Voice Recording, BSS (Best Shot Selector), Blur Warning, Date Imprint, Self-timer (3, 10 sec.)

Movie: 	With sound: TV movie (640*) at 30fps, TV movie (640) at 15fps, Small size (320) at 15fps, Smaller size (160) at 15fps; Sepia movie (320) at 5fps, B/W movie at 15fps; Without sound: Time-lapse movie at 30fps, Electronic VR (Vibration Reduction) available

Capture modes: 	P1: 1) Single, 2) Continuous H (approx. 2.3fps), 3) Continuous L (approx. 2.0fps), 4) Ultra HS (up to 100 pictures at up to 30fps), 5) Multi-shot 16 (16fps 1/16 in size), 6) 5 shot buffer (approx. 1fps), 7) Intvl timer shooting/ P2: 1) Single, 2) Continuous (approx. 2.5fps), 3) Ultra HS (up to 40 pictures at up to 30fps), 4) Multi-shot 16 (16fps 1/16 in size), 5) 5 shot buffer (approx. 1.2fps), 6) Intvl timer shooting

Number of frames:

(w/ internal memory) 	P1: Fine: approx. 7, Normal: approx. 14, Basic: approx. 27 / P2: Fine: approx. 5, Normal: approx. 10, Basic: approx. 20

Built-in flash: 	Range: 0.5-3.8m/ 1' 8" - 12' 6" (W), 0.5-2.0m/ 1' 8" - 6' 7" (T); Flash modes: Auto, Auto with Red-eye Reduction (In-Camera Red-Eye Fix), Flash Cancel, Anytime Flash, Slow sync and Rear Curtain Sync.

Interface: 	Wi-Fi, USB 2.0, Audio Video output

Wireless capability: 	IEEE802.11b/g, wireless range up to 30m (depending on environment)

Supported languages: 	German/English/Spanish/French/Italian/Dutch/Swedish/Japanese/Simplified Chinese/Traditional Chinese/Korean/Russian selectable in menu display

Power requirements: 	One Rechargeable Li-ion Battery EN-EL8 (supplied), AC Adapter EH-62C (optional)

Battery life (approx.): 	P1: Approx. 180 shots with EN-EL8 Rechargeable Battery / P2: Approx. 190 shots with EN-EL8 (based on CIPA standard*)

Dimensions (WxHxD):	Approx. 91 x 60 x 39mm (3.6 x 2.4 x 1.5 in.)

Weight: 	Approx. 170g (6 oz.) (without battery and SD memory card)

Supplied Accessories**:	Rechargeable Li-ion Battery EN-EL8, Battery Charger MH-62, USB Cable, Audio Video Cable, Strap, PictureProject CD-ROM

Optional Accessories: 	AC Adapter EH-62C, Wireless Printer Adapter PD-10

* Industry standard for measuring life of camera batteries. Measured at 25°C

(77°F); zoom adjusted with each shot, built-in flash fired with every other shot,

image mode set to Normal (COOLPIX P1: 8M; COOLPIX P2: 5M).

** Supplied accessories may differ by country or area.

PictureProject System Requirements

OS	Macintosh: Mac® OS X version 10.1.5 or later (version 10.2.8 or later required for Burn Disc option) / Windows: Windows® XP Home Edition/Professional, Windows® 2000 Professional, Windows® Me, Windows® 98SE pre-installed models

RAM	Macintosh: 64MB or more recommended/ Windows: 64MB or more recommended (128MB with muvee option)

Hard disk	60MB required for installation

Display	800 x 600 or more with 16-bit color (full color recommended)

Others	CD-ROM drive required for installation

Only built-in USB ports are supported

Microsoft® and Windows® are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries. Macintosh® and QuickTime® are either registered trademarks or trademarks of Apple Computer Inc. in the United States and/or other countries. The FotoNation logo is a trademark. The SD logo is a trademark. PictBridge is a trademark. D-Lighting technology is provided by Apical Limited. Face-priority AF technology is provided by Identix®. Products and brand names are trademarks or registered trademarks of their respective companies.

Specifications and equipment are subject to change without any notice or obligation on the part of the manufacturer. September 2005 ©2005 NIKON CORPORATION