Nikon Digital Camera COOLPIX S3 Specifications

Type:	Compact digital camera

Effective pixels: 	6.0 million

CCD 	1/2.5-inch type (6.4 million total pixels)

Image modes: 	High (2816*), Normal (2816), Normal (2048), PC (1024), TV (640)

Lens: 	3x Zoom-Nikkor ED; 5.8-17.4mm (35mm [135] format equivalent to 35-105mm); f/3.0-5.4; 12 elements in 10 groups; Digital zoom: up to 4x

Focus range:	30cm (12 in.) to infinity (∞), 4cm (1.6 in.) to infinity (∞) in Macro Mode

LCD monitor: 	2.5-inch type, 110,000-dot TFT LCD monitor with brightness adjustment

Storage media:	Internal memory: approx. 12MB, SD memory card

Shooting modes:	Auto, Scene Assist (Portrait, Landscape, Sports, Night portrait), Scene (Party/Indoor, Beach/Snow, Sunset, Dusk/Dawn, Night landscape, Close up, Museum, Fireworks show, Copy, Back light, Panorama assist, Voice Recording, Underwater), BSS (Best Shot Selector), Color Options, Blur Warning, Date Imprint, Self-timer (10 sec.)

Movie:	With sound: TV movie (640) at 15fps, Small size (320) at 15fps, Smaller size (160) at 15fps; Without sound: Time-lapse movie at 15fps

Capture modes:	1) Single, 2) Continuous (approx. 1.6fps), 3) Multi-shot 16 (16 frames 1/16 in size)

Number of frames

(w/ Internal memory):	6M: High (2816*) approx. 4, Normal (2816) approx. 9, 3M: Normal (2048) approx. 16

Built-in flash:	Range: 0.3-2.5m/ 1 ft. – 8 ft. 2 in. (W), 0.3-1.4m/ 1 ft. – 4 ft. 7 in. (T); Flash modes: Auto, Auto with Red-eye Reduction (In-Camera Red-Eye Fix), Flash cancel, Anytime flash and Slow sync.

Interface

(via COOL-STATION MV-11/MV-11B): USB, Audio Video output

Supported languages:	German/English/Spanish/French/Italian/Dutch/Swedish/Japanese/Simplified Chinese/Traditional Chinese/Korean/Russian selectable in menu display

Power requirements: 	One Rechargeable Li-ion Battery EN-EL8 (supplied), AC Adapter EH-63 (supplied)

Battery life (approx.): 	Approx. 190 shots with Rechargeable Battery EN-EL8 (based on CIPA standard*)

Dimensions (W x H x D):	Approx. 89.9 x 57.5 x 19.7mm (3.5 x 2.3 x 0.8 in.)

Weight:	Approx. 118g (4 oz.) (without battery and memory card)

Supplied accessories:†	COOL-STATION MV-11/MV-11B, Strap, USB Cable UC-E10, Audio Video Cable EG-E5000, AC Adapter EH-63, Rechargeable Li-ion Battery EN-EL8, PictureProject CD-ROM

Optional accessories:	COOL-STATION MV-11/MV-11B, USB Cable UC-E10, Audio Video Cable EG-E5000, AC Adapter EH-63, Rechargeable Li-ion Battery EN-EL8, Battery Charger MH-62, Waterproof Case WP-CP5‡

* Industry standard for measuring life of camera batteries. Measured at 25°C (77°F); zoom adjusted with each shot, built-in flash fired with every other shot, image mode set to Normal.

† Supplied accessories may differ by country or area.

‡ Not available in some countries or areas.

PictureProject System Requirements

OS	Macintosh: Mac® OS X version 10.1.5 or later (version 10.2.8 or later required for Burn Disc option) / Windows: Windows® XP Home Edition/Professional, Windows® 2000 Professional, Windows® Me, Windows® 98SE pre-installed models

RAM	Macintosh: 64MB or more recommended / Windows: 64MB or more recommended (128MB with muvee option)

Hard disk	60MB required for installation

Display	800 x 600 or more with 16-bit color (full color recommended)

Others	CD-ROM drive required for installation

Only built-in USB ports are supported

Microsoft® and Windows® are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries. Macintosh® and QuickTime® are either registered trademarks or trademarks of Apple Computer Inc. in the United States and/or other countries. The FotoNation logo is a trademark. The SD logo is a trademark. PictBridge is a trademark. D-Lighting technology is provided by Apical Limited. Face-priority AF technology is provided by Identix®. Products and brand names are trademarks or registered trademarks of their respective companies.

Specifications and equipment are subject to change without any notice or obligation on the part of the manufacturer. August 2005 ©2005 NIKON CORPORATION